MANNERS

HARTHILL HALL

DERBYSHIRE

FIRE RISK ASSESSMENT

MANNERS

Harthill Hall

Derbyshire

Fire Risk Assessment.

- People at Risk

  - The Owners who undertake maintenance and up-keep. They will use flammable and combustible materials on rare occasions but these are not left on the premises whilst the property is occupied by guests.

  - Paying guests, including children. Guests may have restricted mobility. Mobility limitations are discussed before reservation and opportunities for reasonable adjustment explored.

- Fire Risk

  - Combustibles / Flammables.

    - The fabric of the building is stone, and has exposed timber beams in the lounge. Low level combustibility.

    - The roof supports are timber

    - All living areas including kitchen have blinds with one door curtain (believed to be flame retardant.)

    - The kitchen fittings are wooden with a composite worktop.

    - The bedroom fittings are wooden

    - Bedding is fire retardant.

    - The bathroom fittings are ceramic (toilet and wash-basin), acrylic shower tray with tempered glass shower screen, linoleum covering to floor.

    - Kitchen floor is wooden with linoleum covering and leading to the open plan lounge and bedrooms which carpeted throughout. Lounge includes modern fire resistant rugs.

    - Upholstery is fire retardant.

    - LPG gas supply inspected and checked.

    - Aerosols are used when cleaning, for the duration of the cleaning only.

    - Agents containing flammable solvents needed occasionally e.g. for maintenance.

  - Ignition sources.

    - Electrical installation.

    - Portable electrical appliances in all rooms.

    - Oven, hob, portable cooking appliances i.e. microwave oven and toaster.

- Control Measures.

    - The electrical installation is understood to be to British Standard.

    - The cooker and related equipment are to a good standard and inspected/maintained at each change of occupancy.

    - Portable electrical appliances are checked annually for earth continuity, earth leakage and visual inspection.

    - Agents containing flammable solvents, e.g. for maintenance, are removed from the property after use

    - There is a smoke detector in the kitchen which is tested by the maintenance team on duty at each change of occupancy.

    - Fire extinguisher and fire blanket situated in the kitchen.

    - This risk assessment is scheduled for an annual review.

    - The Emergency Contact Number is: 07703 126635.

- Statement of residual risk.

It is the owners’ assessment that:

- All combustible and flammable materials have been identified.

- All ignitions sources have been identified.

- Satisfactory control measures are implemented and maintained.

- Whilst structural integrity would eventually be compromised in the event of a prolonged fire, control measures, escape routes and warnings are such that the risks to personal safety are minimal.

- For these reasons and subject to up-keep of the control measures, the residual risk is deemed to be acceptable.

Assessed by: L. Miller

Date: 05/05/22

Review due:05/05/23